# 5 Whys Template for 8D D4

Use one chain for why the defect was made (occurrence) and one for why it was not detected (escape). Every answer needs evidence in the third column. The grey prompts can be deleted as you fill in. Your customer's form and customer-specific requirements (CSR) always come first.

| Complaint / 8D no. |  |
| --- | --- |
| Part number / name |  |
| Problem statement (from D2, with numbers) |  |
| Starting fact (verified point of failure) |  |
| Team / date |  |

# Chain A – Occurrence: why was the defect made?

Start from a verified fact (station, tool, lot), not from a guess.

Each answer must be checkable: a record, a measurement or a test.

Stop at a process, method, tool or system cause you can change. Not at a person.

| Why | Answer (a fact, not an opinion) | Evidence / how verified | Status (confirmed / ruled out / open) |
| --- | --- | --- | --- |
| Why 1 |  |  |  |
| Why 2 |  |  |  |
| Why 3 |  |  |  |
| Why 4 |  |  |  |
| Why 5 |  |  |  |

# Chain B – Escape: why was the defect not detected?

Start from the control that should have caught it (the escape point).

Ask why that check missed it: method, sample size, limit sample, gauge capability.

Verify, for example, by running known-bad parts through the check.

| Why | Answer (a fact, not an opinion) | Evidence / how verified | Status (confirmed / ruled out / open) |
| --- | --- | --- | --- |
| Why 1 |  |  |  |
| Why 2 |  |  |  |
| Why 3 |  |  |  |
| Why 4 |  |  |  |
| Why 5 |  |  |  |

# Verification of the root causes

Passive: data, logs, good-vs-bad comparison that match the failure time and place.

Active (where safe and practical): switch the cause on and off and show the defect follows.

A 5 Whys chain finds candidate causes; it does not verify them on its own.

| Root cause | Type (occurrence / escape) | Passive or active check | Result / data |
| --- | --- | --- | --- |
|  | Occurrence |  |  |
|  | Escape |  |  |

# Hand-over to D5

One permanent action for each verified cause, occurrence and escape.

“Retrain the operator” or “add inspection” alone usually means the chain stopped too early.