# 8D Report – Template

Use this form to draft your 8D. The grey prompts say what evidence a reviewer expects in each discipline; delete them as you fill in. Your customer's form and customer-specific requirements (CSR) always come first.

| Complaint / 8D no. |  |
| --- | --- |
| Customer |  |
| Part number / name |  |
| Supplier / site |  |
| Date opened |  |
| Report status / revision |  |
| 8D owner |  |

# D0 – Preparation and emergency response

Evidence to include:

Is an 8D required (customer request, severity, repeat issue)?

Emergency actions already taken, with date and who.

# D1 – Team

Evidence to include:

Team leader, plus production, quality and, where needed, engineering, maintenance or sub-supplier.

Name, function and contact for each member.

| Name | Function / department | Role in team | Contact |
| --- | --- | --- | --- |

# D2 – Problem description

Evidence to include:

What, where, when, how many, how detected – with numbers.

Part number, lot / date codes, shipment window, requirement not met (spec value vs actual).

Is / is-not: where, when and on which parts the defect appears – and where it could appear but does not.

No suspected cause here. D2 describes the deviation; D4 explains it.

|  | Is | Is not |
| --- | --- | --- |
| What |  |  |
| Where |  |  |
| When |  |  |
| How many |  |  |

# D3 – Containment actions

Evidence to include:

Every location: line, WIP, warehouse, in transit, customer site, other sites.

Quantity checked and quantity NOK per location, including zero results.

Sort method and inspection criteria.

Break point (clean point): from which date, lot or serial number parts are good, and how they are marked.

| Location | Qty checked | Qty NOK | Method | Date / owner |
| --- | --- | --- | --- | --- |

# D4 – Root cause

Evidence to include:

Occurrence root cause: why the defect was made. Use 5-Why / Ishikawa and go down to a process, method, tool or system cause.

Escape root cause: why the existing controls did not detect it (the escape point).

Verification of each cause with data: switch the defect on and off, test results, capability data. Not only team opinion.

“Operator error” or “lack of attention” is not a final root cause – ask why the process allowed it.

| Cause | Type (occurrence / escape) | How verified | Result / data |
| --- | --- | --- | --- |

# D5 – Permanent corrective actions

Evidence to include:

At least one action per verified cause – for occurrence and for escape.

Actions change process, tooling, method or detection, not only attention or training.

Planned verification for each action (what data will prove it works).

| Action | Addresses cause | Owner | Due date | Planned verification |
| --- | --- | --- | --- | --- |

# D6 – Implementation and verification of effectiveness

Evidence to include:

Implementation date for each action.

Verification data after implementation: measurements, trial runs, capability, challenge tests with limit samples.

“No complaints since” is not evidence on its own. If data is pending, give the date.

When and how containment (D3) is ended.

| Action | Implemented on | Verification data / result | Status |
| --- | --- | --- | --- |

# D7 – Prevent recurrence

Evidence to include:

PFMEA updated (occurrence / detection ratings reviewed), with revision.

Control plan, work and inspection instructions updated, with revision.

Read-across to similar parts, lines or sites – as a list, not “will be reviewed”.

Lessons learned recorded and shared.

| Document / area | Old rev. | New rev. | Date | Owner |
| --- | --- | --- | --- | --- |

# D8 – Closure and team recognition

Evidence to include:

Propose closure only when actions are implemented and verified, or list open items with dates.

Status in D8 matches D3–D7 (quantities, dates, action status).

Team recognition.

Closure proposed by (supplier 8D owner), date: ______________________