# D2 Is / Is-Not Worksheet

A generic worksheet for the 8D problem description (D2). It describes the problem as observed, not the cause. Check your customer's 8D format and supplier manual for required fields.

# 1. Identification

| Complaint / 8D no. |  | Customer |  |
| --- | --- | --- | --- |
| Part number / name |  | Team / date |  |

# 2. Is / Is-Not table

| Question | IS (where / when it occurs) | IS NOT (could occur but does not) | Difference / what changed | Source / evidence |
| --- | --- | --- | --- | --- |
| WHAT – object / part (which part numbers, variants) |  |  |  |  |
| WHAT – defect (which characteristic, how far out of spec) |  |  |  |  |
| WHERE – seen on the part (location, feature) |  |  |  |  |
| WHERE – found (customer plant, line, field, own inspection) |  |  |  |  |
| WHERE – made (line, machine, tool, cavity, shift) |  |  |  |  |
| WHEN – first seen (date, lot, serial) |  |  |  |  |
| WHEN – history (pattern over time; changes to product, process, material, supplier, people or gauges before or around first seen) |  |  |  |  |
| WHEN – in the life cycle (incoming, assembly, test, field, after how long) |  |  |  |  |
| HOW MANY / HOW MUCH (quantity, rate, trend, size of defect) |  |  |  |  |
| HOW – detected (method, who, why not earlier) |  |  |  |  |

Fill the IS NOT column with a real comparison: a similar part, line, lot or period where the defect could appear but does not. The 'Difference / what changed' column gives the clues for D4. The history row asks when the problem was first seen and what changed before or around that time.

# 3. Problem statement

| Problem statement (one or two sentences, measurable) |  | Open questions / data still needed |  |
| --- | --- | --- | --- |
| Clues passed to D4 |  | Checked by (name, date) |  |

Test: could someone outside the team find a defective part from this statement alone? Suspected causes go to D4, where they are verified.