# PFMEA, control plan and work instruction update after an 8D (D7)

One sheet per 8D. Replace every [bracket] and delete the grey prompts. Generic checklist: your own FMEA method (for example the AIAG-VDA FMEA Handbook or your customer's method), document control and change process come first. Advisory only, not a customer decision.

# 1 – Link to the 8D

| 8D / complaint no. | [no.] |
| --- | --- |
| Part / process | [Part number, rev.], [line / operation no.] |
| D4 occurrence cause | [Why it was made] |
| D4 escape cause | [Why it was not detected] |
| D5 action(s) | [What changed in the process or the check] |
| D6 result | [Verified on (lots / period / volume), result] (update documents only after D6 shows the action works) |
| D7 owner | [Name, function] |

# 2 – Was it in the PFMEA?

☐  Yes: the failure mode and cause were there, but the O or D rating was too optimistic or the control did not work. Why: [ ]

☐  No: the failure mode or cause was missing. Why the FMEA team missed it (scope, lessons learned not used, no data from similar parts): [ ]

☐  Fix for the FMEA process itself, not only this row: [ ]

# 3 – PFMEA change (before / after)

| Item | Before | After |
| --- | --- | --- |
| Failure mode / cause row | [Existing row no. / new row] | [Revised wording] |
| Prevention control | [Before] | [After] |
| Detection control | [Before] | [After] |
| Occurrence (O) | [Before] | [After, re-assessed with D6 data] |
| Detection (D) | [Before] | [After] |
| Severity (S) | [Before] | [Normally unchanged unless the design changes] |
| AP or RPN | [Before] | [After] (use your customer's FMEA method) |
| Document no., rev., date | [Old rev.] | [New rev., date, reviewed by] |

# 4 – Control plan change

| Characteristic | [Feature / process parameter] |
| --- | --- |
| Method / gauge | [Gauge, fixture, test] (new gauge? measurement system analysis done: [ref.]) |
| Sample size and frequency | [Before] -> [After] |
| Reaction plan | [What the operator does on a failed check, who is called, what is held] |
| D3 extra check | [Stays as permanent control / ends on (date) because (D6 evidence)] |
| Control plan no., rev., date | [New rev., date] |

# 5 – Documents changed

| Document | No. | Revision | Date | Note |
| --- | --- | --- | --- | --- |
| Work instruction | [Doc no.] | [Old rev.] -> [New rev.] | [Date] | [How operators were told: briefing record ref.] |
| Setup sheet / parameter sheet | [Doc no.] | [ ] -> [ ] | [Date] | [ ] |
| Inspection instruction | [Doc no.] | [ ] -> [ ] | [Date] | [ ] |
| Maintenance / tool-life plan | [Doc no.] | [ ] -> [ ] | [Date] | [ ] |
| Drawing / specification | [Doc no.] | [ ] -> [ ] | [Date] | [Design change? who owns it] |
| Training matrix | [Doc no.] | [ ] -> [ ] | [Date] | [Which people, which revised document] |

Delete rows that do not apply. A document that did not need to change can stay, marked 'checked, no change', with the reason.

# 6 – Customer and change control

☐  Checked whether this process change must be notified to the customer, or needs their agreement first, under their customer-specific requirements (for example PPAP or a change notification rule). Result: [ ]

☐  Change recorded in the internal change / document control system: [ref.]

☐  Other plants or functions told (design, purchasing, maintenance): [ ]

# 7 – Read-across

| Checked | Why it could have the same cause | Result | Action |
| --- | --- | --- | --- |
| [Line / product / tool / plant] | [Same cause possible? why] | [Present / not present / n.a.] | [Action, owner, date] |
| [ ] | [ ] | [ ] | [ ] |
| [ ] | [ ] | [ ] | [ ] |

# 8 – Ready for D8?

☐  D6 showed the action works before the documents were changed.

☐  Every changed document has a number, revision and date, and an extract is attached to the 8D.

☐  The escape side is covered: a detection change, not only an occurrence change, if the escape point was part of D4.

☐  Lessons learned recorded where the next FMEA team will find them: [ref.]

☐  Open items listed with owner and planned date, not marked as done.